Residential Plan Checklist

Use this checklist to review your building plans. All references to IRC 2021 Code

Square Footage for:

| Main Floor: |  |  | Carport/Covered Deck: |  |
| --- | --- | --- | --- | --- |
| Second Floor |  |  | Deck: |  |
| Rough Basement: |  |  | Remodel: |  |
| Finished Basement: |  |  | Addition: |  |
| Garage: |  |  | Other: |  |

|  | APPLICATION FOR PERMIT: description of work, contractor’s license numbers or owner builder, signed | CKCOG Website |
| --- | --- | --- |
|  | CLIMATIC AND GEOGRAPHICAL DESIGN CRITERIA | CKCOG Website (Climate Zone 5) |
|  | EXTERIOR WALLS AND OPENINGS: fire separation distance from property lines | R302 pg. 3-3 |
|  | MINIMUM AREA DIMENSIONS AND HEIGHTS FOR HABITALE SPACES: 70 ft/2 floor space. Not < 7 feet in any horizontal dimension. Minimum ceiling height is 7’. Bathrooms, toilet rooms and laundry rooms can have a ceiling height of 6’-8”. | R304 & R305 pg. 3-39 – 3-41 |
|  | GLAZING IN HAZARDOUS LOCATIONS: bathtubs, showers, within 24 inches of doors, within 18 inches of floors and greater than 9 sq ft, within 60 inches of stairway landings, within 36 inches of walkways and within 60” from waters edge of tubs and showers. | R308 pg. 3-41 – 3-45 |
|  | GARAGE SEPARATION FROM DWELLING: 1/2-inch gypsum wall board between the garage and residence, drywall to extend to gable peak or the residence sidewall and garage ceiling can be covered with 1/2-inch drywall. Garage beneath habitable rooms shall be separated from all habitable rooms by no less than 5/8-inch Type X gypsum board or equivalent on ceiling, 1/2-inch gypsum on all supporting structure. Openings between the garage and residence shall be equipped with a 1 3/8-inch-thick solid or a 20-minute fire rated door | R302 Table R302.6 pg. 3-37 |
|  | EMERGENCY ESCAPE AND RESCUE OPENINGS: 44 inches max above floor, in every sleeping room, minimum 5.7 sq ft, 20-inch min width, 24-inch min height. Exception: Grade floor or below grade openings not < 5 sq ft. | R310 pg. 3-46 – 3-47 |
|  | WINDOW WELLS AND LADDERS: 9 sq ft min area, 36 inches out from window, if deeper than 44 inches a permanently affixed ladder or steps usable with the window fully open. | R310.4 pg. 3-46 – 3-47 |
|  | EXIT DOORS AND HALLWAYS: At least one 32 x 80 exit door is required. Hallways shall have a minimum width of 36 inches. | R311.2 & R311.6 Pg. 3-48 – 3-49 |
|  | LANDINGS AT DOORS AND STAIRWAYS: 36 inches minimum in the direction of travel and must be as wide as the door. | R311.3 & R311.7.6 pg. 3-48 – 3-49 |
|  | STAIRWAY WIDTH: 36 inches, TREADS AND RISERS: 8 1/4-inch max rise and 9-inch minimum tread, HEADROOM: 6-foot 8 inch, UNDER STAIR PROTECTION: ½ inch sheetrock, HANDRAILS: four (4) or more risers, 34 to 38 inches in height and 1 ¼ to 2 5/8 inches in diameter if circular | R311.7 pg. 3-49 – 3-49 |

|  | GUARDRAILS (GUARDS): at floors over 30 inches above grade, 36 inches min. height, a 4-inch sphere shall not pass through | R312 pg.3-51 – 3-52 |
| --- | --- | --- |
|  | SMOKE DETECTORS: inside each sleeping room, outside sleeping rooms, on each level, all wired in series with battery backup | R314 pg. 3-52 – 3-53 |
|  | CARBON MONOXIDE ALARMS: approved carbon monoxide detector installed outside of each sleeping area in the immediate vicinity of the bedrooms when a fuel-burning appliance is installed in dwelling units or with attached garages. | R315 pg. 3-53 – 3-54 |
|  | CONCRETE FOOTING SIZES AND DEPTH: SEE TABLES 403.1 (1), (2), (3) AS APPLICABLE. | R403 pg. 4-2 – 4-6 |
|  | ANCHOR BOLTS TO FOUNDATION: max spacing 72 inches on center & not more than 12 inches from each end of plate section and or corners. Anchor straps may be substituted for bolts as long as they provide equivalent anchorage. | R403.1.6 pg. 4-10 |
|  | STRUCTURAL COLUMNS: 4x4 inch min. wood, 3-inch minimum pipe, restrained to prevent lateral displacement at the bottom. | 407 pg. 4-4 |
|  | UNDER-FLOOR VENTILATION: 1 sq ft for each 150 sq ft of underfloor space or 1 sq ft for every 1500 sq ft if covered with a class 1 vapor retarder. | R408 pg.4-44 – 4-45 |
|  | UNDER-FLOOR ACCESS: 24 inch by 18 inch | R408.4 pg. 4-45 |
|  | FLOOR FRAMING: double under bearing partitions, blocking at bearing walls, allowable spans | R502 pg. 5-1 - 5-8 |
|  | HEADER & GIRDER SPANS. TABLES: 602.7(1), 602.7(2), 602.7(3). | R607 pg. 6-14 – 6-20 |
|  | WALL CONSTRUCTION | R602 Pg. 6-1 |
|  | BRACED WALL LINES AND PANELS: within 12.5 feet of wall corner, 4 feet in length for plywood with nailing at 6 inches on center at edges and 12 inches on center in field, 8 feet for sheetrock with screws at 7 inches on center edges and field | R602.10 Pg. 136 (2006 IRC) |
|  | FASTENER SCHEDULE: | TABLES R602.3(1) & R602.3(2) pg. 6-3 – 6-7 |
|  | EXTERIOR WALL COVERINGS AND WEATHER BARRIERS: | R703 pg. 7-7 – 7-28 |
|  | ROOF FRAMING: truss lay-out, RIDGES: 1x with width to match rafter cuts, PURLINS: 4 foot on center max. and 8 feet max length, CONNECTIONS & RAFTER TIES: at 4 feet max on center, hurricane ties (H-1), ROOF SHEATHING, and size and spacing of rafters at OVER-BUILDS | R802 pg. 8-1 |
|  | ROOF VENTILATION: The minimum net free ventilation area shall be 1/150 of the area of the vented space with exceptions (only one per the 2015 IRC) | R806 pg. 8-46 |
|  | ACCESSIBLE ATTIC ACCESS: 22 x 30 inches with 30 inches of vertical clearance above | R807.1 Pg. 8-48 |

|  | ICE DAM PROTECTION: ice and water shield at eaves to 24 inches inside the exterior wall line of the building | R905.1.2 Pg. 9-2 |
| --- | --- | --- |
|  | INSULATION VALUES IN WALLS, FLOORS, CEILINGS, WINDOWS, AND DOORS: Basement walls = R-15/19, Crawl Space Walls = R-15/19, Walls = R- 23 cavity/R-13 + 7.5ci/R-20 + 3.8ci, Floors = R-30, Ceilings = R-49, Windows = U- 0.30 OR RES-CHECK 2021 IECC. | N1102.1 w/RAC amendment or 2025 PA ALTERNATIVE RESIDENTIAL ENERGY PROVISIONS |
|  | BLOWER DOOR TESTING (Mandatory) | N1102.4 pg. 11-46 |
|  | DUCT TESTING (Mandatory) (Exception: Unless air handler and all ducts located entirely within conditioned space) | N1103.3.3 pg. 11-50 |
|  | MECHANICAL VENTILATION REQUIRED | N1103.6 pg. 11-51 |
|  | CLOTHES DRYER EXHAUST: 35 feet to outside with 5-foot reduction for 90-degree bends | M1502 pg. 15-1 |
|  | MECHANICAL VENT TERMINATIONS: vent terminal shall be located not less than 4 feet below, 4 feet horizontally from, or 1 foot above any door, window or gravity air inlet into a dwelling. | M1804.2 - M1804.2.6 pg. 18-3 – 18-4 |
|  | DWV SYSTEM: Riser diagram showing plumbing fixtures, trap, and vent sizes. Vent terminations per Chapter 31. | P3005 Pg. 30-6 -30-9 |
|  | ELECTRICAL EQUIPMENT LOCATIONS & CLEARANCES: | E3405 pg. 34-4 -34-5 |
|  | ILLUMINATION PER SECTION 3405.7 | Pg. 34-4 |
|  | REQUIRED RECEPTACLE OUTLETS IN: walls, counters, islands, floors, bathroom, outdoors, laundry, basement, garage, hallways, and mechanical room | E3901 pg. 39-1 -39-3 |
|  | ARC-FAULT CIRCUIT-INTERRUPTER PROTECTION IN: bedrooms, kitchens, family rooms, dining rooms, living rooms, parlors, dens, hallways, rec rooms, laundry rooms and similar areas | E3902.16 Pg. 39-4 -39-5 |
|  | GFCI PROTECTION IN: All 125 volt -250 volt receptacles in bathroom and jetted tub motors, garage, outdoor, crawlspace, basement, kitchen counters, islands, dishwashers and bar sinks and electrically heated floors and any receptacle within 6’ of sink edge. | E3902 pg. 39-5 |
|  | RECEPTACLES REQUIREMENTS: TAMPER RESISTANT: | E4002 E4002.14 Pg. 40-3 – 40-4 |